AM Form 031

4th Edition WEF: 21 Nov’17

INSPECTION REPORT

CATEGORY N2 - N3 (VC 8025 Ed 6)

| Cop or Homologation |  |
| --- | --- |
| Dealership or MIB Name |  |
| Date of inspection/homologation |  |
| Name and surname of inspector |  |
| Signature of inspector |  |

# SAMPLE VEHICLE DETAILS

| Make |  |
| --- | --- |
| Model |  |
| Description |  |

WHEN MEASUREMENTS ARE TAKEN FROM GROUND LEVEL VEHICLE WITH AIR SUSPENSION MUST BE IN OPERATIONAL MODE

1. REAR LIGHTS (CLAUSE 3.1.1& 3.1.2)

|  | Requirements | Stop |  | Rear position |  | Rear direction indicator |  | Reverse |  | Rear Fog Lamp (Optional) |  | High Mounted Stop Lamp (O) |  | Rear end outline marker lamps(O) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Height from ground | 350-1500mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 250-1200mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance from outer edge | Not more than 400mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Distance apart | Not <600mm |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Geometric visibility(inwards) |  | 45º |  | 45º |  | 45º |  | 45º |  | 25º |  | 45º |  |  |
|  |  |  |  |  |  |  |  | 30º |  |  |  |  |  |  |
| Geometric visibility(outwards |  | 15º |  | 80º |  | 80º |  | 45º |  | 25º |  | 45º |  | 80º |
| Geometric visibility(upwards) |  | 15º |  | 15º |  | 15º |  | 15º |  | 5º |  | 15º |  | 5º |
|  |  |  |  | 5º |  |  |  |  |  |  |  |  |  |  |
| Geometric visibility(downward |  | 15º |  | 15º |  | 15º |  | 5º |  | 5º |  | 15º |  | 20º |
|  |  |  |  |  |  |  |  |  |  |  |  | 5º |  |  |
| Visibility 30M |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Brand Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Markings | (Tick) or (Circle) | S1 /S2/ S3 |  | R/R-S1/ R-S2/ R-S3 |  | 2a/ 2b |  | AR |  | F |  | S3 |  | R |
| SABS/Approval number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Colour& Amount fitted | (Tick) or (Circle | Red& 2 |  | Red &2 |  | Amber& 2 |  | White &1/2 |  | Red &1/2 |  | Red&1/2 |  | Red &2 |
| C/NC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Note 1: 	If one fog lamp is fitted it shall be fitted in the middle or right hand side of vehicle

The distance between fog lamp and stop lamp shall be greater than 100mm

Fog lamps shall only operate when the dipped beam or main beam is switched on.

Front and rear fog lamps shall be capable of being switched on or off independently.

Note 2:   All measurements must be taken from ground or outer edge to the lower/outer edge of the

illuminating surface.

Note 3:   All measurements must be taken from ground/outer edge to the centre of the reflector

2. REAR REFLECTORS (Clause 3.1.2)

|  | Requirements | Measurement | Result C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | 300-1500mm |  |  | Close as possible |
| Distance from outer edge | Not more than 400mm |  |  |  |
| Position of reflector | Vertical and face squarely to rear |  |  |  |
| Visibility | 30M |  |  |  |
| Markings | 1 or 1A or 1B |  |  |  |
| Colour and amount | Red & 2 |  |  |  |
| Brand name of rear reflectors |  |  |  |  |
| E Marking/SABS marking |  |  |  |  |
| Additional information/comments | The height measurement must be measured to the center of the reflector |  |  |  |

3. NUMBER PLATE LAMPS (Clause 3.1.1 and 3.1.2)

|  | Requirements | Measurement | Results C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | Max 1500mm |  |  |  |
| Colour | White |  |  |  |
| Brand name of number plate lamp/s |  |  |  |  |
| Amount of lamps fitted |  |  |  |  |
| E Marking/SABS marking |  |  |  |  |
| Additional information/comments |  |  |  |  |

Note: Only provision to install registration plate as per reg 35 of NRTA shall apply. Holes on plate are not necessary a requirement.

4. CHEVRON BOARD (only if GVM exceeds 3500kg) (Clause 3.1.3)

|  | Requirements | Measurement | Result C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | Not more than 1100mm |  |  | May be higher |
| Distance from outer edge | Not more than 400mm |  |  |  |
| Chevron board width | 200mm ± 5mm |  |  |  |
| Geometric visibility | Upright or 15º thereof |  |  |  |
| Markings | Certification mark |  |  |  |
| Colour | Red &Yellow |  |  |  |
| Both red and yellow strips shall be retro reflective |  |  |  |  |
| Name of chevron board |  |  |  |  |
| E Marking/SABS marking |  |  |  |  |
| Additional information/comments | Measure height from ground level to lower edge for the chevron board |  |  |  |

5. REAR UNDERRUN PROTECTION DEVICE (Clause 3.8)

(Truck tractor, chassis cabs, vehicles with a GVM of less than 8000kg, vehicles for which rear under run device are incompatible with their use and vehicles of which the rear extremity of the rear tyres and the rear surface of the structure is less than 450mm are exempt)

|  | Requirements | Measurement | Results C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | Not more than 550mm |  |  | To lower edge |
| Length at rear (Max) | Not wider than tyres and axle |  |  |  |
| Length at rear (Min) | Not more than 100mm on either side |  |  |  |
| Sectional height | Not less than 100mm |  |  |  |

Note: All measurements must be taken from ground/outer edge to the lower/outer edge of the underrun device.

6. REAR YELLOW REFLECTIVE TAPE (Clause 3.1.4)

(Goods vehicles of GVM 3500kg or less and chassis cabs are exempt)

|  | Requirements | Measurement | Result C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | 250-1500mm |  |  | 2100mm max |
| Distance from outer edge | Calculate 80% |  |  | 80% of width |
| Visibility | 30M |  |  |  |
| Markings | Trade name |  |  |  |
| Marking | “C” |  |  |  |
| Marking | Approval number |  |  |  |
| Colour | Reflective Yellow |  |  |  |
| *The distance shall not exceed more than 600mm from the lowest part of the body |  |  |  |  |
| The distance shall not be less than 200mm from any stop lamp |  |  |  |  |
| Yellow reflective tape shall be 50mm + 10mm – 0mm wide |  |  |  |  |
| Brand name of contour marking tape |  |  |  |  |
| E Marking/SABS marking |  |  |  |  |
| Additional information/comments |  |  |  |  |

* Note: Take two measurements, from the body to the reflective tape and from ground to the lower edge of the reflective tape.

7. SIDE YELLOW REFLECTIVE TAPE (Clause 3.1.4)

(Chassis cabs and goods vehicles that have a GVM of 3500kg or less and which are less than 7m in length are exempt)

|  | Requirements | Measurement | Result C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | 250-1500mm |  |  | 2100mm max |
| Side Coverage | At least 80% |  |  |  |
| Visibility | 30M |  |  |  |
| Markings | Trade name |  |  |  |
| Marking | “C” |  |  |  |
| Marking | Approval number |  |  |  |
| Colour | Reflective Yellow |  |  |  |
| The distance shall not exceed more than 600mm from the lowest part of the body |  |  |  |  |
| Yellow reflective tape shall be at least 50mm + 10mm – 0mm wide |  |  |  |  |
| Name of contour marking tape |  |  |  |  |
| E Marking/SABS marking |  |  |  |  |
| Additional information/comments |  |  |  |  |

Note: All measurements must be taken from the ground to the lower edge of the reflective tape.

8. TYRES (Clause 6.4)

Axle 1

| Tyre make |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tyre size |  |  |  |  |  |  |
| E mark |  |  |  |  |  |  |
| Load rating | Load rating single |  | Load rating dual |  |  |  |
| GVM of vehicle |  |  |  |  |  |  |
| Speed rating |  |  |  |  |  |  |
| Total number of axles fitted with dual tyres |  |  |  |  |  |  |
| Total number of axles fitted with single tyres |  |  |  |  |  |  |
| Total carrying capacity of all tyres in kilograms |  |  |  |  |  |  |
| Requirements for tyres |  |  |  | Result C/NC |  | Remark |
| Does the total carrying capacity of all the tyres equal or exceed the GVM of the vehicle |  |  |  |  |  | See above |
| Does speed rating on tyre equal or exceed the maximum speed indicated on NVM form |  |  |  |  |  |  |

Axle 2

| Tyre make |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tyre size |  |  |  |  |  |  |
| E mark |  |  |  |  |  |  |
| Load rating | Load rating single |  | Load rating dual |  |  |  |
| GVM of vehicle |  |  |  |  |  |  |
| Speed rating |  |  |  |  |  |  |
| Total number of axles fitted with dual tyres |  |  |  |  |  |  |
| Total number of axles fitted with single tyres |  |  |  |  |  |  |
| Total carrying capacity of all tyres in kilograms |  |  |  |  |  |  |
| Requirements for tyres |  |  |  | Result C/NC |  | Remark |
| Does the total carrying capacity of all the tyres equal or exceed the GVM of the vehicle |  |  |  |  |  | See above |
| Does speed rating on tyre equal or exceed the maximum speed indicated on NVM form |  |  |  |  |  |  |

Axle 3

| Tyre make |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tyre size |  |  |  |  |  |  |
| E mark |  |  |  |  |  |  |
| Load rating | Load rating single |  | Load rating dual |  |  |  |
| GVM of vehicle |  |  |  |  |  |  |
| Speed rating |  |  |  |  |  |  |
| Total number of axles fitted with dual tyres |  |  |  |  |  |  |
| Total number of axles fitted with single tyres |  |  |  |  |  |  |
| Total carrying capacity of all tyres in kilograms |  |  |  |  |  |  |
| Requirements for tyres |  |  |  | Result C/NC |  | Remark |
| Does the total carrying capacity of all the tyres equal or exceed the GVM of the vehicle |  |  |  |  |  | See above |
| Does speed rating on tyre equal or exceed the maximum speed indicated on NVM form |  |  |  |  |  |  |

Load rating table

| Load Rating | Carrying capacity (kg) | Number of tyres | Total carrying capacity |  | Load Rating | Carrying capacity (kg) | Number of tyres | Total carrying capacity |  | Load Rating | Carrying capacity (kg) | Number of tyres | Total carrying capacity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 93 | 650 |  |  |  | 126 | 1700 |  |  |  | 159 | 4375 |  |  |
| 94 | 670 |  |  |  | 127 | 1750 |  |  |  | 160 | 4500 |  |  |
| 95 | 690 |  |  |  | 128 | 1800 |  |  |  | 161 | 4625 |  |  |
| 96 | 710 |  |  |  | 129 | 1850 |  |  |  | 162 | 4750 |  |  |
| 97 | 730 |  |  |  | 130 | 1900 |  |  |  | 163 | 4875 |  |  |
| 98 | 750 |  |  |  | 131 | 1950 |  |  |  | 164 | 5000 |  |  |
| 99 | 775 |  |  |  | 132 | 2000 |  |  |  | 165 | 5150 |  |  |
| 100 | 800 |  |  |  | 133 | 2060 |  |  |  | 166 | 5300 |  |  |
| 101 | 825 |  |  |  | 134 | 2120 |  |  |  | 167 | 5450 |  |  |
| 102 | 850 |  |  |  | 135 | 2180 |  |  |  | 168 | 5600 |  |  |
| 103 | 875 |  |  |  | 136 | 2240 |  |  |  | 169 | 5800 |  |  |
| 104 | 900 |  |  |  | 137 | 2300 |  |  |  | 170 | 6000 |  |  |
| 105 | 925 |  |  |  | 138 | 2360 |  |  |  | 171 | 6150 |  |  |
| 106 | 950 |  |  |  | 139 | 2430 |  |  |  | 172 | 6300 |  |  |
| 107 | 975 |  |  |  | 140 | 2500 |  |  |  | 173 | 6500 |  |  |
| 108 | 108 |  |  |  | 141 | 2575 |  |  |  | 174 | 6700 |  |  |
| 109 | 1030 |  |  |  | 142 | 2650 |  |  |  | 175 | 6900 |  |  |
| 110 | 1060 |  |  |  | 143 | 2725 |  |  |  | 176 | 7100 |  |  |
| 111 | 1090 |  |  |  | 144 | 2800 |  |  |  | 177 | 7300 |  |  |
| 112 | 1120 |  |  |  | 145 | 2900 |  |  |  | 178 | 7500 |  |  |
| 113 | 1150 |  |  |  | 146 | 3000 |  |  |  | 179 | 7750 |  |  |
| 114 | 1180 |  |  |  | 147 | 3075 |  |  |  | 180 | 8000 |  |  |
| 115 | 1215 |  |  |  | 148 | 3150 |  |  |  | 181 | 8250 |  |  |
| 116 | 1250 |  |  |  | 149 | 3250 |  |  |  | 182 | 8500 |  |  |
| 117 | 1285 |  |  |  | 150 | 3350 |  |  |  | 183 | 8750 |  |  |
| 118 | 1320 |  |  |  | 151 | 3450 |  |  |  | 184 | 9000 |  |  |
| 119 | 1360 |  |  |  | 152 | 3550 |  |  |  | 185 | 9250 |  |  |
| 120 | 1400 |  |  |  | 153 | 3650 |  |  |  | 186 | 9500 |  |  |
| 121 | 1450 |  |  |  | 154 | 3750 |  |  |  | 187 | 9750 |  |  |
| 122 | 1500 |  |  |  | 155 | 3875 |  |  |  | 188 | 10000 |  |  |
| 123 | 1550 |  |  |  | 156 | 4000 |  |  |  | 189 | 10300 |  |  |
| 124 | 1600 |  |  |  | 157 | 4125 |  |  |  | 190 | 10600 |  |  |
| 125 | 1650 |  |  |  | 158 | 4250 |  |  |  | 191 | 10900 |  |  |

Speed rating table

| Speed category symbol | Corresponding speed Km/h |  | Speed category symbol | Corresponding speed Km/h |
| --- | --- | --- | --- | --- |
| F | 80 |  | Q | 160 |
| G | 90 |  | R | 170 |
| J | 100 |  | S | 180 |
| K | 110 |  | T | 190 |
| L | 120 |  | U | 200 |
| M | 130 |  | H | 210 |
| N | 140 |  | V | 240 |
| P | 150 |  | W | 270 |

9. WHEEL FLAPS (Clause 6.5)

(Optional for truck tractors and chassis cabs)

| Requirements |  | Result C/NC | Remark |
| --- | --- | --- | --- |
| The horizontal distance between the wheel flap and the tyre shall not exceed 300mm |  |  |  |
| The bottom edge of the wheel flap and the road shall not exceed 200mm |  |  |  |
| Wheel flap shall be equal or more than the breadth of the tyre/s |  |  |  |
| Wheel flap shall be fitted if the distance between two tyres exceed 350mm |  |  |  |
| Shall be no space between wheel flap and body or wheel flap and wheel guard |  |  |  |
| Additional information/comments |  |  |  |

10. SIDE DIRECTION INDICATORS (Clause 3.1.1 and 3.1.2)

|  | Requirements | Measurement | Results C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | 500-1500mm |  |  | 2300max |
| Distance from front of vehicle | Not more than 1800mm |  |  | 2500 max |
| Geometric visibility (inward) | 45º |  |  |  |
| Geometric visibility (outwards) | Between 5º- 60º |  |  | Angles from body outwards |
| Geometric visibility (upwards) | 15º |  |  |  |
| Geometric visibility (downwards) | 15º or 5º |  |  |  |
| Visibility | 30M |  |  |  |
| Markings | 5 or 6 |  |  |  |
| Colour and amount | Amber & 2 |  |  |  |
| Brand name of indicator lamp |  |  |  |  |
| E Marking/SABS marking |  |  |  |  |
| Additional information/comment |  |  |  |  |

11. FRONT RETRO REFLECTORS (Optional) (Clause 3.1.2)

|  | Requirements | Measurement | Result C/NC | Remark |
| --- | --- | --- | --- | --- |
| Height from ground | 350-900mm |  |  | 1500mm max |
| Distance from outer edge | Not more than 400mm |  |  |  |
| Width apart | Not less than 600mm |  |  | 400mm min |
| Geometric visibility (inward) | 30º |  |  |  |
| Geometric visibility (outwards) | 30º |  |  |  |
| Geometric visibility (upwards) | 15º |  |  |  |
| Geometric visibility (downwards) | 15º or 5º |  |  |  |
| Visibility | 30M |  |  |  |
| Markings | 1 or 1A |  |  |  |
| Colour and amount | White & 2 |  |  |  |
| Brand name of front reflectors |  |  |  |  |
| E Marking/SABS marking |  |  |  |  |
| Additional information/comments |  |  |  |  |

12. REAR VIEW MIRRORS (Clause 3.2.1)

| Interior rear view mirror (Class I) |  | Close proximity mirrors (Class V) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Brand name |  | Brand name |  |  |  |
| E Marking/SABS marking |  | E Marking/SABS marking |  |  |  |
| Main exterior mirrors (Class II mirrors) Mandatory |  | #### Wide angle mirrors (Class IV) |  |  |  |
| Brand name |  | Brand name |  |  |  |
| E Marking/SABS marking |  | E Marking/SABS marking |  |  |  |
| Requirements |  |  | Results C/NC |  | Remark |
| All rear view mirrors shall be adjustable |  |  |  |  |  |
| All edges of mirrors shall have a radius of curvature of 2.5mm |  |  |  |  |  |
| Mirrors shall not obstruct the driver view when the driver is in the normal seated position |  |  |  |  |  |
| Rear view mirrors of which the lower edge of the mirror are less than 2000mm from ground level, shall not protrude more than 200mm from the edge of the bodywork of the vehicle |  |  |  |  |  |
| The interior rear view mirror shall be capable of being adjusted by the driver from his/her driving position |  |  |  |  |  |
| The exterior rear view mirror situated on the drivers side shall be capable of being adjusted from inside the vehicle the driver’s side window may be open. |  |  |  |  |  |
| Two class II mirrors shall be fitted one on the left and one on the right of the vehicle |  |  |  |  |  |
| Class II mirrors hall have a breakaway feature both forward and rearward |  |  |  |  |  |

13. FRONT LIGHTS (CLAUSE 3.1.1& 3.1.2)

|  | Requirements | Main Beam Head Lamps | Dipped Beam Headlamps |  | Front direction indicator |  | Front position lamps |  | Front Fog Lamp (Optional) |  | Front end outline marker lamps(O) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Height from ground | 350-1500mm |  |  |  |  |  |  |  |  |  |  |
|  | 250-1200mm |  |  |  |  |  |  |  |  |  |  |
|  | 500-1200mm |  |  |  |  |  |  |  |  |  |  |
|  | Higher than windscreen |  |  |  |  |  |  |  |  |  |  |
| Distance from outer edge | Not more than 400mm | Inside of dipped beam |  |  |  |  |  |  |  |  |  |
| Distance apart | Not <600mm |  |  |  |  |  |  |  |  |  |  |
| Geometric visibility(inwards) |  |  | 10º |  | 45º |  | 45º |  | 10º |  |  |
| Geometric visibility(outwards |  |  | 45º |  | 80º |  | 80º |  | 45º |  | 80º |
| Geometric visibility(upwards) |  |  | 15º |  | 15º |  | 15º |  | 5º |  | 5º |
| Geometric visibility(downward |  |  | 10º |  | 15º |  | 15º |  | 5º |  | 20º |
|  |  |  |  |  | 5º |  | 5º |  |  |  |  |
| Visibility 30M |  |  |  |  |  |  |  |  |  |  |  |
| Arrow Facing to the right |  |  |  |  |  |  |  |  |  |  |  |
| Brand Name |  |  |  |  |  |  |  |  |  |  |  |
| Markings | (Tick) or (Circle) | HR | HC |  | 1/ 1a/1b |  | A |  | F |  | A |
| SABS/Approval number( markings) |  |  |  |  |  |  |  |  |  |  |  |
| Colour & Amount fitted | (Tick) or (Circle | White 2/ 4 | White & 2 |  | Amber & 2 |  | White/ Yellow&2 |  | White &1/2 |  | White &2 |
| C/NC |  |  |  |  |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |  |  |  |  |

Notes

HR: marked on headlamps means H for a halogen lamp and R for main beam

HC: marked on headlamps mean H for halogen and C for dipped beam.

If lamp is marked HCR then (This lamp is capable of emitting a dipped beam and a main beam S may also be added to this marking indicating that the lamp is sealed

Fog lamp must not be higher than headlamp

All measurements must be taken from ground or outer edge to the lower/outer edge of the

illuminating surface.

14. WINDSCREEN wipers (Clause 3.2.3)

| Requirements | Results C/NC |
| --- | --- |
| At least one windscreen wiper shall be fitted that is capable of operation other than manual |  |
| The windscreen wiper blade when in operation shall wipe the outside of the windscreen directly in front of the driver evenly and efficiently |  |
| Additional comments |  |

| Windscreen mark |  |  |  |
| --- | --- | --- | --- |
| Manufacturers logo |  |  |  |
| E Marking/SABS marking |  |  |  |
| Additional markings |  |  |  |
| Requirements |  | Results C/NC | Remark |
| Manufacturers name or logo shall appear on windscreen |  |  |  |
| The word laminated or the mark “//” shall be on windscreen |  |  |  |
| Additional comments |  |  |  |

15. WINDSCREEN (Clause 3.2.2.)

16. WINDOWS AND PARTITION (Clause 3.2.2.1)

| Glass location |  | Glass location |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Manufacturers name |  | Manufacturers name |  |  |  |
| Manufacturers trade mark |  | Manufacturers trade mark |  |  |  |
| Additional markings |  | Additional markings |  |  |  |
| E Marking/SABS marking |  | E Marking/SABS marking |  |  |  |
| Additional comments: |  | Additional comments |  |  |  |
| Glass location |  | Glass location |  |  |  |
| Manufacturers name |  | Manufacturers name |  |  |  |
| Manufacturers trade mark |  | Manufacturers trade mark |  |  |  |
| Additional markings |  | Additional markings |  |  |  |
| E Marking/SABS marking |  | E Marking/SABS marking |  |  |  |
| Additional comments |  | Additional comments |  |  |  |
| ### Requirements |  |  | Results C/NC |  | ### Remark |
| The wording toughened or tempered shall be marked on the safety glass as well as the manufacturers name or trade mark(not mandatory for glass with E mark) |  |  |  |  |  |
| All glass shall be clearly identified as safety glass and a permanent mark shall be applied to the glass identifying it as such |  |  |  |  |  |
| If plastic glazing material is used instead of safety glass this material shall not be to the immediate left or right of the driver |  |  |  |  |  |
| Any plastic glazing material shall comply with the requirements of SABS 1472 and a test report shall be submitted to prove such compliance |  |  |  |  |  |

| Requirements |  |  |  |  |  |  |  |  |  | Actual data |  |  |  |  |  | Remark |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VIN shall be stamped on right hand side of vehicle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| When stamped on the vehicle chassis the VIN shall be at least 7mm high |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VIN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

17. VIN NUMBER (Clause 5.2.5)

18. ENTRANCES (Clause 3.5)

| Requirements |  | Result C/NC | Remark |
| --- | --- | --- | --- |
| Every entrance or exit shall be fitted with a door or an effective barrier |  |  |  |
| Doors shall have a double latch system |  |  |  |
| Additional information/comments |  |  |  |

19. DATA PLATES (Clause 5.2.1)

| Requirements |  |  |  |  |  |  |  |  |  |  |  |  | Results C/NC |  |  |  |  | Remarks |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| One or two data plates shall be affixed to the vehicle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The data plates shall be riveted or welded or affixed by any other method which will achieve permanency of attachment during the life of the vehicle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Data plates shall be attached in a conspicuous position to a doorpost or to a panel in the engine compartment, or to the dash panel or in the vehicle cab. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The height of the roman letters and Arabic numeral shall be 3mm when marked on the data plate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The data plate information shall be legibly and permanently stamped or imprinted on such data plate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| The Roman letters in the VIN shall not contain I, O or Q. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Requirements |  | Actual data |  |  |  |  |  |  |  |  |  | Remarks |  |  |  |  |  |  |  |  |  |
| GVM/BVM kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GCM/BKM kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GA/BA Front kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GA/BA Rear kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GAU/BAE (for axle units) kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| P/D (Net power) kw |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A Front kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A Rear kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AU/AE (for axle units) kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DT (Permissible maximum drawing vehicle mass) kg |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Vehicle identification number (VIN) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional information | The abbreviation for the prefixes are not required if they appear in the order as follows: GVM GCM GA or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GAU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

20. SPEEDOMETERS (Clause 6.1)

| Maximum speed indicated |  |  |  |
| --- | --- | --- | --- |
| Graduation 5 10 20 km/h |  |  |  |
| Multiples of 20 km/h |  |  |  |
| Clearly legible Day or night |  |  |  |
| Requirements |  | Results C/NC | Remark |
| Vehicle shall be marked predominantly in km/h |  |  |  |

21. STEERING (Clause 3.4.1.2 and 3.4.1.3)

| Requirements |  | Results C/NC | Remark |
| --- | --- | --- | --- |
| A vehicle shall be of a right hand drive configuration or center steering configuration |  |  |  |
| Additional comments |  |  |  |

22. CONTROLS (Clause 3.3.1)

| Requirements |  | Results C/NC | Remark |
| --- | --- | --- | --- |
| All controls that are fitted to a vehicle and that are required for the operation of the vehicle shall be so located that the driver can reach and operate them when he is seated in the normal driving position with his seat belt (if any) fastened. |  |  |  |
| Additional comments |  |  |  |

23. AUDIBLE WARNING DEVICES (Clause 3.4.2)

| Requirements |  | Results C/NC | Remark |
| --- | --- | --- | --- |
| A vehicle shall be fitted with one or more audible warning devices |  |  |  |
| When they are operated a continuous sound of at least 93db shall be produced |  |  | Test report required |
| Additional comments |  |  |  |

24. SEATS (Clause 3.6)

| Seat make |  |
| --- | --- |
| Number of seating positions |  |
| Additional comments |  |

25. RESTRAINING DEVICES (safety belts) (Not in compulsory)

| Location in vehicle | Drivers safety belt | Additional safety belt | Additional safety belt |
| --- | --- | --- | --- |
| Manufacturers logo |  |  |  |
| E Marking/SABS marking |  |  |  |
| Test report number |  |  |  |
| Part number |  |  |  |
| Identification marking |  |  |  |
| Lap belt or three point belt |  |  |  |
| COMMENT 1: No restraining devices are required to be fitted to vehicles the GVM which exceeds 3500kg but if fitted the following requirements shall apply |  |  |  |
| COMMENT 2: No restraining devices are required to be fitted to the rear rows of seats of the box type construction of vehicles that contain at least three rows of seats |  |  |  |

26. ELECTRICAL CONNECTORS (Clause 3.7)

| Requirements |  | Results C/NC | Remark |
| --- | --- | --- | --- |
| Electrical connectors fitted for the purpose a towed vehicle shall comply with the requirements of SABS 1327:1981 (7 pole connectors) or SABS ISO 1146:1993 (13 pole connectors) for 12V systems |  |  |  |
| Electrical connectors fitted for the purpose a towed vehicle shall comply with the requirements of SABS ISO 12098:1994 For 24V systems |  |  |  |
| Additional comments |  |  |  |

27. EXHAUST SYSTEM (Clause 6.2.2)

| Requirements |  | Result C/NC | Remark |
| --- | --- | --- | --- |
| No part of the exhaust system shall be in such a position that oil or flammable liquid or material can drip or fall onto it. |  |  |  |
| The exhaust shall not be so positioned that exhaust gas leaks into the driving cab. |  |  |  |
| Additional information/comments |  |  |  |

28. ENGINE (Clause 6.2.1)

| An engine of a vehicle shall be so provided with a cover that any part of the engine that constitutes a source of danger is out of reach of a normal person | Results C/NC | Remark |
| --- | --- | --- |
| Engine Number: |  |  |

29. TRANSMISSION (Clause 6.2.3)

| Requirements |  | Results C/NC | Remark |
| --- | --- | --- | --- |
| A self-propelled vehicle shall be equipped with a transmission that enables it to be controlled and driven in both the forward and reverse direction. |  |  |  |
| Additional comments |  |  |  |

30. FUEL SYSTEMS (Clause 6.3)

| Requirements |  | Results C/NC | Remark |
| --- | --- | --- | --- |
| The orifice for filling a fuel tank on a vehicle shall be fitted with an effective cap that prevents incidental ingress of water or other foreign matter |  |  |  |
| Additional comments |  |  |  |

31. BRAKES AND BRAKING EQUIPMENT (Clause 3.3)

| Requirements |  |  |  | Results C/NC | Remark |
| --- | --- | --- | --- | --- | --- |
| At least two reliable controls |  |  |  |  |  |
| A check point shall be situated at rearmost air chamber furthest from the foot brake |  |  |  |  |  |
| A check point shall be situated before the load sensing valve |  |  |  |  |  |
| A circuit low air warning device shall be fitted it shall be visual and audible |  |  |  |  |  |
| The braking system shall comply with the requirements of ECE R13/SANS 20013 |  |  |  |  |  |
| Additional comments |  |  |  |  |  |
| ABS Fitted |  | Yes | No |  |  |
| Short description of Braking System fitted |  |  |  |  |  |

32. VEHICLE DIMENSIONS (Clause 5.1)

|  | Requirements |  | Measurement | Results C/NC | Remark |
| --- | --- | --- | --- | --- | --- |
| Overall width of vehicle | Not more than 2600mm |  |  |  |  |
| Overall height of vehicle | Not more than 4300mm |  |  |  |  |
| Overall length of vehicle | Not more than 15000mm |  |  |  |  |
| Turning radius | Not more than 13100mm |  | Self certified |  |  |
| Wheelbase | Not more than 8500mm |  |  |  |  |
| Front overhang | Not more than 60% of wheelbase |  |  |  |  |
| Rear overhang | Not more than 60% of wheelbase |  |  |  |  |
| Additional information/comments |  |  |  |  |  |

33. SUMMARY OF NON COMPLIANCES

OBSERVATIONS:

Responsible Person Signature………………………………..				Date:………………………….

CORRECTIVE ACTION

| Summary of non-compliant | Detailed Description of corrective action | Date Corrected |
| --- | --- | --- |

Inspector signature:………………………………..				Date:…………………………………………